#

File: hitachi.info,  Node: Top,  Prev: (grundig.info),  Next: (jvc.info),  Up: (remotes.info)

******************************************************************************

HITACHI

******************************************************************************

* Menu:

* Technical info::

* Video::

#

File: hitachi.info,  Node: Technical info,  Prev: Top,  Next: Video,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:            32 bits

Carrier:                39.2kHz

The code is only transmitted once, and after that a stop code is transmitted.

+--------------------------------+

Header:      |                                |

+                                +----------------+..

16T                       8T

8.8ms                     4.4ms

+--+

1 is coded:  |  |

..+  +------+..

T   3T

+--+

0 is coded:  |  |

..+  +--+..

T  T

+--+

|  |

After the last bit a ( ..+  +  ) is transmitted to terminate the last bit.

T

+--------------------------------+        +--+

Stop code:    |                                |        |  |

+                                +--------+  +

16T                  4T     T

8.8ms                2.2ms

T = 550us

Space between transmissions: 95000us between the headers of the codes.

To use the code (not with the remote package):

GROUP * 10000h + (Code * 100h) + (Code XOR FFh)

For more information please read the documentation.

#

File: hitachi.info,  Node: Video,  Prev: Technical info,  Up: Top

******************************************************************************

Video

******************************************************************************

Video1:

GROUP   #06F9

Video2:

GROUP   #0679

Code    Function

----    --------

#50     Rew

#28     Play

#90     FF

#D0     Stop

#58     Pause

#28     Rec

#E8     Operate

#A2     Index

#E2     Repeat

#0C     SP/LP

#08     Step

#80     Tracking -

#00     Tracking +

#C0     Slow -

#88     Slow

#40     Slow +

#62     Counter remain

#82     Counter reset

#22     Counter memory

#02     Osd menu

#42     Osd disp

#30     0

#B0     1

#70     2

#F0     3

#38     4

#B8     5

#78     6

#F8     7

#20     8

#A0     9

#18     Ch -

#98     Ch +