#

File: grundig.info,  Node: Top,  Prev: (goldstar.info),  Next: (hitachi.info),  Up: (remotes.info)

******************************************************************************

GRUNDIG

******************************************************************************

* Menu:

* Technical info::

* TV::

* Video::

* Digital receiver::

#

File: grundig.info,  Node: Technical info,  Prev: Top,  Next: TV,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:            10 bits

Header: 500us + 2600us

One:    space + pulse   pulse = 500us, space = 550us

Zero:   pulse + space

Space after precode: 18ms

Space between transmissions: 120ms

The code is transmited as precode + code + postcode.

Precode is always #000h transmited once.

Post code is always #000h transmited twice.

To use it with TRANSMIT you need 3 infostrings:

GRUNDIG, GRUNDIG2 and GRUNDIG3.

If you want to make a remote program the strings are used like this:

(This needs the code on stack as a binary)

\<<

GRUNDIG2 #000h TRANSMIT @ transmits the pre code

GRUNDIG SWAP TRANSMIT   @ transmits the actual code

GRUNDIG3 #000h TRANSMIT @ Transmits the post code

\>>

For more information please read the documentation

#

File: grundig.info,  Node: TV,  Prev: Technical info,  Next: Video, Up: Top

******************************************************************************

TV

******************************************************************************

No GROUP var is needed.

Code    Function

----    --------

#001    OK

#003    mute

#005    AV

#009    contr+

#00B    contr-

#011    +

#013    -

#015    bright+

#017    bright-

#019    color+

#01B    color-

#021    0/AV

#023    1

#025    2

#027    3

#029    4

#02B    5

#02D    6

#02F    7

#031    8

#033    9

#035    P/C

#055    P+

#057    P-

#05B    i

#From: Stefan_Wolfrum@su.maus.de (Stefan Wolfrum)

Hi folks,

recently I found out all codes for all functions on a Grundig TV remote control

to be used by Remote 3.2. I just have a German remote control available and

don't know the English inscriptions - but the layout would be the same. Where

possible I used English terms. Here you are:

+----+----+----++-----+-----+---------++-----------+

|1   |2   |3   ||MIX  |AKT  |SPEICHERW|| - COLOR + |

|  23|  25|  27||   7B|   79|       7D||1B   :   19|

+----+----+----++-----+-----+---------++-----------+

|4   |5   |6   ||S+1/M|GROSS|ANTW.    || - BRIGH + |

|  29|  2B|  2D||   73|   75|       69||17   :   15|

+----+----+----++-----+-----+---------++-----------+

|7   |8   |9   ||STOP |INH. |VORW.    || - TREBL + |

|  2F|  31|  33||   77|   71|       5D||F    :    D|

+----+----+----++-----+-----+---------++-----------+

|0/AV|10  |20  ||CLOCK|     |TEXT/TV  || - BASS  + |

|  21|  35|  37||   5B|     |       61||B    :    9|

+----+----+----++-----+-----+---------++-----------+

|<o> |xxxx|.\. ||>|<  |HIFI |SPEAK.OFF|| - VOLUM + |

|  39|xxxx|   7||    1|   3D|        3||13   :   11|

+----+----+----++-----+-----+---------++-----------+

|    |    |    ||     |     |         ||    OFF    |

|    |    |    ||     |     |         ||          5|

+----+----+----++-----+-----+---------++-----------+

Legend:

+-----+

function ->|CLOCK|

|   5B|<- hexadecimal code

+-----+

Have fun.

Stefan.

File: grundig.info,  Node: Video,  Prev: TV,  Next: Digital receiver, Up: Top

******************************************************************************

Video

******************************************************************************

No GROUP var is needed.

Code    Function

----    --------

#115    Play

#119    Stop

#131    FF

#12F    REW

#117    Pause

#11B    Rec

#12B    Ch+

#129    Ch-

#123    Rew search

#125    FF search

The following codes are taken from a Grundig video with VPT textprogramming.

Kjetil Arve Dahle (kjetild@ulke.dhmolde.no)

#127	Slow

#1a1	0

#1a3	1

#1a5	2

#1a7	3

#1a9	4

#1ab	5

#1ad	6

#1af	7

#1b1	8

#1b3	9

#1db	Clock

#1e1	INFO    Choose the infomenu.

#1f7	AV

#191	Right/+

#193	Left/-

#1dd	Down

#1f1	Up

#1e5	OK

#1f5	Magnify (texttv)

File: grundig.info,  Node: Digital receiver,  Prev: Video,  Up: Top

******************************************************************************

Digital receiver

******************************************************************************

No GROUP var is needed.

Code            Function         (tested on a Grundig DSR 100)

------------------------------------------------------------------------

#201            OK

#203            mute

#205            power

#211            volume +

#213            volume -

#223            1

#225            2

#227            3

#229            4

#22B            5

#22D            6

#22F            7

#231            8

#233            9

#235            10

#237            11

#239            select *)

#241            12

#243            13

#245            14

#247            15

#249            16

#24B            info

#255            program +

#257            program -

*) This function toggles between program selection (then the keys 1..16

choose a channel) and program type selection (then you can select a

type like news, rock music, sports etc with the numeric keys).

These codes were provided by Frank Heepmann